Here are the three alignment specs we have for Mustangs:

Street

Front camber -1.03 degrees on both sides (Performance and Handling Pack)

Front camber -.72 degrees on both sides (non-Performance and Handling Pack)

Caster +7.0 degrees on both sides

Front toe +0.01 degrees per side total toe of +0.02 degrees, this is toe in

Rear camber -1.5 degrees on both sides

Rear toe +0.115 degrees per side, total toe of +0.23 degrees, this is toe in

DRAG

This is drag race only.

Front camber -0.75 degrees, must be even on both sides.

Caster +7.0 (+/- .25 degrees), must be even on both sides.

Front toe +0.01 per side, total toe +0.02. This toe in.

Rear camber -0.5 degrees (+/- .25 degrees), must be even on both sides.

Rear toe +0.12 degree, must be even on both sides. This toe in.

Track

Typical track/autocross alignment settings(not street use);

Front camber -3.0 degrees on both sides

Caster +7.0 degrees on both sides

Front toe +0.01 degrees per side total toe of +0.02 degrees, this is toe in

Rear camber -1.6 to -1.7 degrees on both sides

Rear toe +0.12 degrees per side, total toe of +0.24 degrees, this is toe in